# Complete F1TENTH Advanced Racing Stack — AI Generation Prompt

**Copy and paste this entire prompt** into any AI code generation tool (Claude, GPT-4, Gemini, etc.) to generate the complete ROS 2 Humble package.

## PROMPT START

You are an expert autonomous robotics engineer specializing in F1TENTH scaled autonomous racing.

Your task is to generate a complete, production-quality ROS 2 Humble Python package called f1tenth_race_stack from scratch. This is inspired by the ForzaETH race_stack architecture (https://github.com/ForzaETH/race_stack).

Do NOT copy ForzaETH code. Implement everything from scratch using their architecture as a structural reference only.

## 1. HARDWARE SPECIFICATION (DO NOT CHANGE THESE VALUES)

Extract these exact values and embed them into the vehicle_params.yaml config file and all relevant algorithms:

| Parameter | Value |
| --- | --- |
| **Chassis** | Traxxas Slash 4x4 VXL Ultimate |
| **Drivetrain** | 4WD |
| **Wheelbase** | 0.324 m |
| **Width** | 0.296 m |
| **Length (total)** | 0.568 m |
| **Mass** | 3.0 kg |
| **Motor** | Traxxas Velineon 3500 Kv |
| **Battery** | Bonka U2 Series 3S LiPo, 5200 mAh |
| **ESC / Drive** | VESC MK-VI |
| **Remote Control** | RadioMaster MT12 |
| **Computing Platform** | Intel NUC 12 Pro (Intel i7 12th Gen, **NO GPU**) |
| **LiDAR** | 2D Planar LiDAR (e.g., Hokuyo UST-10LX or similar) |
| **Tyre Outer Diameter** | 100 mm |
| **Estimated Max Speed** | ~10 m/s (Velineon 3500Kv on 3S LiPo) |
| **Max Steering Angle** | 0.43 radians (~25 degrees physical limit) |

## 2. OVERALL PACKAGE STRUCTURE

Generate a single ROS 2 Humble ament-python package with exactly this folder structure:

f1tenth_race_stack/
├── package.xml
├── setup.py
├── setup.cfg
├── README.md
│
├── config/
│   ├── vehicle_params.yaml       # Physical car params from datasheet above
│   ├── mppi_params.yaml          # All MPPI tuning params (FULLY dynamic)
│   ├── ftg_params.yaml           # Follow The Gap params (FULLY dynamic)
│   └── slam_params.yaml          # SLAM Toolbox cartographer params
│
├── launch/
│   ├── mapping_joystick.launch.py     # Mode 1: Manual joystick SLAM mapping
│   ├── mapping_autonomous.launch.py   # Mode 2: Autonomous FTG SLAM mapping
│   ├── racing.launch.py               # Mode 3: Full MPPI racing mode
│   └── system_id.launch.py            # Mode 4: System identification drive
│
├── f1tenth_race_stack/
│   ├── __init__.py
│   │
│   ├── mapping/
│   │   ├── __init__.py
│   │   └── follow_the_gap.py          # Autonomous mapping driver (FTG)
│   │
│   ├── racing_line/
│   │   ├── __init__.py
│   │   └── racing_line_generator.py   # Extracts optimal racing line from map
│   │
│   ├── controller/
│   │   ├── __init__.py
│   │   └── mppi_controller.py         # MPPI racing controller (CPU-optimized)
│   │
│   ├── state_machine/
│   │   ├── __init__.py
│   │   └── race_state_machine.py      # Stack-level FSM (MAPPING, RACING, EMERGENCY)
│   │
│   └── utils/
│       ├── __init__.py
│       ├── vesc_bridge.py             # VESC AckermannDrive publisher
│       └── visualizer.py             # RViz2 MarkerArray publisher for debugging
│
└── maps/                             # Saved .pgm and .yaml maps go here

## 3. MODULE-BY-MODULE SPECIFICATION

### 3.1 `config/vehicle_params.yaml`

This file is the single source of truth for all vehicle dimensions. All nodes must read from this file at startup.

vehicle:
  wheelbase: 0.324
  width: 0.296
  length: 0.568
  mass: 3.0
  max_steering_angle: 0.43
  max_speed: 10.0
  min_speed: 0.5
  tyre_diameter: 0.100

topics:
  scan: "/scan"
  odom: "/ego_racecar/odom"
  drive: "/drive"
  map: "/map"
  joy: "/joy"

### 3.2 `config/mppi_params.yaml`

Every single parameter in this file MUST be declared as a ROS 2 dynamic parameter (using declare_parameter in the node). This allows live tuning via ros2 param set or rqt_reconfigure WITHOUT restarting the node.

mppi:
  # Rollout Configuration
  K: 1000          # Number of parallel trajectory rollouts (tuned for Intel i7, no GPU)
  T: 20            # Prediction horizon (number of steps)
  dt: 0.05         # Time step per rollout step [s]

  # Exploration / Temperature
  lambda: 0.1      # Temperature: low = exploit best trajectory, high = more exploration
  
  # Control Noise Standard Deviations
  noise_std_speed: 1.0         # m/s
  noise_std_steering: 0.15     # radians

  # Cost Function Weights
  weight_reference_track: 10.0    # Penalize lateral deviation from racing line
  weight_speed_reward: 1.0        # Reward high speed (negative cost)
  weight_obstacle_penalty: 500.0  # Penalize trajectories near LiDAR-detected obstacles
  weight_heading: 2.0             # Penalize heading misalignment with racing line
  weight_boundary: 200.0          # Penalize trajectories that go outside track boundary
  weight_control_smoothness: 0.1  # Penalize jerky control inputs

  # Safety
  obstacle_clearance_m: 0.35   # Minimum distance to obstacle before max penalty kicks in

### 3.3 `config/ftg_params.yaml`

Also fully dynamic via ROS 2 parameters:

ftg:
  max_speed: 3.0             # [m/s] Conservative speed for safe mapping
  min_speed: 0.5             # [m/s] Minimum speed while steering sharply
  bubble_radius: 0.40        # [m] Safety bubble radius around closest obstacle (car_width/2 + margin)
  max_lidar_range: 10.0      # [m] Cap LiDAR readings at this range
  gap_selection: "largest"   # "largest" or "furthest" - how to select the best gap
  steering_gain: 1.0         # Multiplier for the raw steering output

### 3.4 `mapping/follow_the_gap.py`

Purpose: Drive the car autonomously around an unknown track at safe speed while SLAM Toolbox runs in the background to build a map.

Implementation Requirements:

Subscribe to /scan (sensor_msgs/LaserScan)

All parameters must be read from ftg_params.yaml using declare_parameter / get_parameter

Implement a `param_callback` using `add_on_set_parameters_callback` so that changing any parameter via ros2 param set is applied instantly without restarting the node

Algorithm:

- Preprocess: Replace NaN/Inf with max_lidar_range, clip all values

- Safety Bubble: Find closest point index, zero out a bubble_radius-sized angular arc around it (calculate arc width using arcsin(bubble_radius / closest_dist))

- Gap Finding: Split the range array by zero-gaps, find the longest contiguous non-zero segment

- Best Point: Within the best gap, select the index of the maximum range value

- Steer: Convert best index to angle using angle_min + (best_idx * angle_increment)

- Speed: Dynamically reduce speed based on steering magnitude: speed = max_speed * max(min_speed/max_speed, 1.0 - abs(steer_angle)/(pi/4))

Publish to /drive (ackermann_msgs/AckermannDriveStamped)

### 3.5 `controller/mppi_controller.py`

Purpose: High-speed racing controller using Model Predictive Path Integral (MPPI) control.

CRITICAL CONSTRAINT: The Intel NUC 12 Pro has NO GPU. The entire MPPI must run on the CPU. Use NumPy vectorized operations and matrix broadcasting to simulate all K=1000 rollouts in parallel without Python loops over rollouts.

Implementation Requirements:

Subscriptions:

- /scan (sensor_msgs/LaserScan) — for real-time obstacle detection

- /ego_racecar/odom (nav_msgs/Odometry) — for current state (x, y, yaw, speed)

- /racing_line (nav_msgs/Path) — reference racing line from racing_line_generator

Publication:

- /drive (ackermann_msgs/AckermannDriveStamped)

Dynamic Parameters:

- ALL parameters from mppi_params.yaml must be declared and live-reloadable via add_on_set_parameters_callback

Kinematic Bicycle Model (vectorized over K rollouts):

# State shape: (K, 3) -> [x, y, yaw] for all K rollouts simultaneously
   # u_seq shape: (K, T, 2) -> [speed, steering] for all rollouts and all time steps
   
   def rollout(self, state, u_seq):
       # state: (3,) current [x, y, yaw]
       # u_seq: (K, T, 2)
       x   = np.full(self.K, state[0])
       y   = np.full(self.K, state[1])
       yaw = np.full(self.K, state[2])
       trajectory_costs = np.zeros(self.K)
       
       for t in range(self.T):
           v     = u_seq[:, t, 0]   # (K,)
           delta = u_seq[:, t, 1]   # (K,)
           
           x   += v * np.cos(yaw) * self.dt
           y   += v * np.sin(yaw) * self.dt
           yaw += (v / self.wheelbase) * np.tan(delta) * self.dt
           
           # --- Cost accumulation at each step ---
           # 1. Distance to nearest racing line point
           dists = np.sqrt((self.racing_line[:, 0:1] - x)**2 + (self.racing_line[:, 1:2] - y)**2)
           nearest_dist = np.min(dists, axis=0)  # (K,)
           trajectory_costs += self.weight_reference_track * nearest_dist
           
           # 2. Speed reward (race fast)
           trajectory_costs -= self.weight_speed_reward * v
           
           # 3. Obstacle cost from LiDAR (pre-computed occupancy grid)
           trajectory_costs += self._obstacle_cost(x, y)
       
       return trajectory_costs

MPPI Weight Update:

beta = np.min(costs)
   weights = np.exp(-1.0 / self.lambda_ * (costs - beta))
   weights /= np.sum(weights) + 1e-9  # Avoid division by zero
   self.u_sequence = np.sum(weights[:, None, None] * U, axis=0)

Obstacle Cost from 2D LiDAR:

- Convert the 1D LaserScan into a set of 2D obstacle points in the car's local frame

- For each rollout state (x, y), compute the minimum Euclidean distance to any obstacle point

- Apply a cost that goes to infinity as distance → obstacle_clearance_m

- Use vectorized cdist-style computation via NumPy broadcasting

Receding Horizon: After publishing, shift u_sequence by 1 step forward and warm-start the next iteration

Timer: Run the main MPPI loop at 20 Hz using create_timer(0.05, self.mppi_loop)

### 3.6 `racing_line/racing_line_generator.py`

Purpose: After mapping is complete, generate the optimal racing line (centerline with speed profile).

Implementation:

Load the saved .pgm / .yaml map from the maps/ directory via ROS 2 parameter map_path

Extract the track centerline by:

- Converting OccupancyGrid to a binary image

- Computing the distance transform (scipy.ndimage.distance_transform_edt)

- Following the ridge (skeleton) of the distance transform as the centerline

Smooth the centerline using a moving average filter

Compute a speed profile: reduce target speed at high-curvature points using: v_target = v_max * (1 - curvature_gain * curvature)

Save the racing line as a CSV file: x, y, heading, target_speed

Publish the racing line on /racing_line as nav_msgs/Path for the MPPI controller and for RViz2 visualization

### 3.7 `state_machine/race_state_machine.py`

Purpose: High-level state machine that controls which module is active. Inspired by ForzaETH's stack_master.

States:

IDLE: Car is stationary. Waiting for command

MAPPING_MANUAL: Joystick commands forwarded directly to VESC. SLAM running in background

MAPPING_AUTO: Follow The Gap node is active. SLAM running in background

RACING: MPPI controller is active. Racing line loaded

EMERGENCY_STOP: Override all motion commands. Publish zero velocity at 50 Hz

Transitions:

Subscribe to a /race_mode topic (std_msgs/String) to switch states at runtime

On entering EMERGENCY_STOP, immediately publish a zero-speed AckermannDriveStamped message for 2 seconds

### 3.8 `utils/vesc_bridge.py`

Purpose: Bridge between /drive (ackermann_msgs/AckermannDriveStamped) and the VESC MK-VI driver topics.

Subscribe to /drive

Convert steering_angle and speed into VESC-compatible topics:

- /commands/motor/speed (std_msgs/Float64) — converted using erpm_gain parameter

- /commands/servo/position (std_msgs/Float64) — converted using steering_to_servo_gain and steering_to_servo_offset

All conversion parameters must be dynamic via declare_parameter

vesc_bridge:
  erpm_gain: 4614.0            # ERPM per m/s (tune via system_id)
  steering_to_servo_gain: -0.64  # Tune via system_id
  steering_to_servo_offset: 0.5  # Center position of servo

### 3.9 Launch Files

#### mapping_joystick.launch.py

Launches:

slam_toolbox (online async mode, saving map to maps/)

joy_node (RadioMaster MT12 via /dev/input/js0)

teleop_twist_joy → vesc_bridge (manual drive)

race_state_machine in MAPPING_MANUAL state

rviz2 with a pre-configured .rviz file showing /map, /scan, /tf

#### mapping_autonomous.launch.py

Launches:

slam_toolbox (online async mode)

follow_the_gap node

race_state_machine in MAPPING_AUTO state

rviz2

#### racing.launch.py

Launches:

slam_toolbox (localization-only mode, loads saved map)

racing_line_generator node (loads racing line CSV)

mppi_controller node

vesc_bridge node

race_state_machine in RACING state

rqt_reconfigure — automatically opens the dynamic parameter GUI so the engineer can live-tune MPPI weights while the car is racing on the track

#### system_id.launch.py

Launches:

A simple open-loop drive node that drives the car at fixed speeds and records /odom data

Used to calibrate erpm_gain and steering servo parameters

## 4. DYNAMIC PARAMETER TUNING (CRITICAL REQUIREMENT)

This is a mandatory feature inspired by how ForzaETH tunes their stack on the car.

Every tunable parameter in every node must:

Be declared using self.declare_parameter('param_name', default_value) in __init__

Be read at runtime using self.get_parameter('param_name').value

Have a parameter callback registered via self.add_on_set_parameters_callback(self.param_callback)

The param_callback must update the node's internal state immediately

This means: While the car is autonomously driving on the track, an engineer on a laptop can run:

ros2 param set /mppi_controller mppi.weight_reference_track 15.0
ros2 param set /mppi_controller mppi.K 1500
ros2 param set /mppi_controller mppi.lambda 0.05

...and the MPPI controller will use the new values on the very next control cycle without any restart.

Alternatively, the racing.launch.py file automatically opens rqt_reconfigure with a GUI for all parameters.

## 5. DEPENDENCIES (package.xml and setup.py)

The package must declare these ROS 2 Humble dependencies:

ROS 2 Packages:

rclpy

sensor_msgs

nav_msgs

ackermann_msgs

std_msgs

geometry_msgs

tf2_ros

tf2_geometry_msgs

slam_toolbox

joy

teleop_twist_joy

rcl_interfaces (for parameter callbacks)

Python Libraries (requirements in setup.py extras):

numpy

scipy (for distance transform in racing line generator)

opencv-python (for map image processing)

matplotlib (for offline racing line visualization)

## 6. CODE QUALITY REQUIREMENTS

Every Python file must have full docstrings explaining what the class/function does

Every algorithm step must have inline comments explaining the math

No magic numbers — every constant must come from a parameter or a named variable

Use Python type hints on every function signature

Every node must use self.get_logger().info(...) at startup to confirm it loaded parameters correctly

The mppi_controller.py must log the computation time of each MPPI iteration using Python's time.perf_counter() to help the user verify it runs under 50ms on the Intel i7

## 7. README.md REQUIREMENTS

Generate a complete README.md that includes:

Hardware BOM (Bill of Materials) from the datasheet above

Software dependencies and installation steps for ROS 2 Humble

Step-by-step workflow: How to map the track → Generate racing line → Race

A table of every parameter in every YAML file, what it does, and its default value

How to do live parameter tuning during a race using ros2 param set

## PROMPT END

Generate all files listed in the structure above. Output every file completely — do not skip any file or truncate any code. Start with package.xml, then setup.py, then the config/ YAML files, then all Python source files, and finally the launch files.